Grism Syslog Fields

Revision History

| No. | When | Who | What |
| --- | --- | --- | --- |
| 0.1 | 2020/10/21 | Jay | 新增Matched Log sipv6,dipv6,filter_id,matched_content欄位 |
| 0.2 | 2021/12.24 | Jay | 新增System HeartBeat miss, Drop Packets , and common type Log |
| 0.3 | 2022/04/18 | Jay | 新增subtype以及input packets drop log |
| 0.4 | 2023/06/13 | Jay | 新增 system alert boot, port link up/down , cpu usage, temperature, fan |
| 0.5 | 2023/07/24 | Jay | 新增dnslog dtype_num, inport, SOA等欄位 |
| 0.6 | 2025/03/20 | Jay | 新增 type mec, gtpc, dnsqps, ja4說明以及調整說明 |
| 0.7 | 2026/01/09 | Sena | 新增syslog sample |
| 0.8 | 2026/01/26 | Jay | 更新dnslog |
| 0.9 | 2026/03/11 | Jay | 新增matched log find_type欄位說明 |
| 1.0 | 2026/05/19 | Jay | 新增dnslog opcode, update_op, record, rcode欄位，移除dtype_num欄位 |

Grism Syslog types

| Setting | type | Define |
| --- | --- | --- |
| Syslog | 1 | Matched |
|  | 3 | System |
|  | 7 | MEC |
|  | 9 | GTPC |
|  | 10 | DNSQPS |
|  |  | Common |
| DPI Log | 4 | DNS |
|  | 5 | HTTP |
|  | 6 | SSL |

Syslog ->Matched

<134>datetime=2023-05-23 16:43:02,type=1,sip=106.107.190.22,dip=203.68.0.32,sport=24391,dport=60023,protocol=6,find_id=10044001,filter_id=10001,find_content=106.107.190.22"

<134>datetime=2023-05-22 08:49:19,type=1,sip=125.229.209.141,dip=122.116.229.84,sport=50682,dport=53,protocol=17,find_id=377,filter_id=10002,find_content=qrqnswerqs.info

| Field | Description | 備註 | Format |
| --- | --- | --- | --- |
| type | Grism Syslog type | 1 | Int |
| datetime | Date Time | Ex. 2018-08-08 17:15:10 | yyyy-mm-dd hh:mm:ss |
| sip | Source IP Address | Ex. 192.168.1.10 | Ip address |
| dip | Destination IP Address | Ex. 120.113.1.10 | Ip address |
| sipv6 | Source IPv6 Address | Ex. 402:7500:55e:19ca:b460:360a:2f00:a688 | IPV6 address |
| dipv6 | Destination IPv6 Address | Ex. 2a00:1450:4009:11::a | IPV6 address |
| sport | Source Port | Ex. 1111 | short int |
| dport | Destination Port | Ex. 443 | short int |
| protocol | IP Protocol |  | short int |
| find_id | Grism find id | 通常用作黑名單id使用 | int |
| filter_id | Grism filter id | Ex. 10000 | int |
| find_type | Grism find type | Ex. 其他可參考 https://packetx.gitbook.io/grism-xml/readme/filter/find 71: IPv4 hash 72: dns query name hash 73: dns query name public suffix hash 74: http request url hash | int |
| find_content | Grism Find Content | 通常用作黑名單content使用，內容包含IP, domain, url | string |
| matched_content | Matched content | 當比對條件有hash過或是部分內容比對，例如比對條件是*.googlevideo.com 而matched content是r2---sn-ufohvc-u2xee.googlevideo.com，就會另外再送出個欄位 | String/IP address |

Syslog -> System

<14>Grism:INFO:.192.168.1.100.packetx.logout.

<14>Grism:INFO:.192.168.1.100.packetx.logged.in

<11>Grism:ERROR:.192.168.1.100.admin.Username.or.password.is.invalid.

<6>2026-01-01T12:59:01+08:00.(none).Grism[1301]:.packetx.set.config.

<6>2026-01-01T12:59:01+08:00.(none).Grism[1301]:.packetx.reload.config.

<6>2026-01-01T13:01:04+08:00.(none).Grism[1301]:.packetx.set.config.timeServer.192.168.1.100.

<6>2026-01-01T13:01:04+08:00.(none).Grism[1301]:.packetx.submit.run.xml.

<6>2026-01-01T13:01:04+08:00.(none).Grism[1301]:.packetx.submit.run1.xml.

| Field | Description | 備註 | Format |
| --- | --- | --- | --- |
| type | Grism Syslog type | 3 | Int |
| datetime | Date Time | Ex. 2018-08-08 17:15:10 | yyyy-mm-dd hh:mm:ss |
| subtype | System subtype | 0: input packet drop 1: output packet drop 2: system alert | int |
| HeartBeat miss |  |  |  |
| send_port | Heartbeat Send port | P6 | String |
| receive_port | Heartbeat Receive Port | P7 | String |
| heartbeat_miss | Heartbeat miss | 1 | Int |
| Drop Packets |  |  |  |
| interface | Interface | P6 | String |
| drop_packets_linkdown | Drop packets because link down | 10 | Int |
| drop_packets_outofmem | Drop packets because out of memory | 10 | Int |
| drop_packets_pkofail | Drop packets befause PKO fail | 10 | int |
| Input drop packets |  |  |  |
| packets | Drop packets | 10 | Int |
| Mbps | Mbit per second | 500 | int |
| Pps | Packets per second | 12345 | Int |
| cpu_load_average | Cpu loading | 36,36,36 | string |
| mem_usage | Memory usage | 100/200 | string |
| Alert |  |  |  |
| flows_percent | Flow percent over 90% | Ex. 95% | % |
| flows | Flow number | Ex. 64000/65535 | Int/int |
| Boot | System boot | Ex. 1 | 1 |
| port_status_name | Port status name | Ex. P0 | string |
| port_status_link | Port status link DOWN/UP | Ex. DOWN | String |
| cpu_usage | CPU usage over 95% | Ex. 99.88% | Double |
| temperature_overheat | Temperature overheat | Ex. 85.25 C | string |
| fan_fault_index | FAN fault | Ex. 1 | int |

Syslog -> MEC

Syslog for Mobile Edge Computing

Syslog -> GTPC

Syslog for GTP control plane protocol

Syslog -> DNSQPS

Syslog for dns client query per second

Syslog -> Common

Syslog for使用者登入web介面的操作

Format: Grism: level: clientip user did what

Example

Mar 20 11:14:04 192.168.1.153 Grism: INFO: 192.168.1.228 packetx logout

Mar 20 11:14:09 192.168.1.153 Grism: INFO: 192.168.1.228 packetx logged in

DPI log->DNS

DNS一次query & response 會是兩筆紀錄如下，目前不會關連成一筆，事後關聯可以用client,server,client_port,server_port欄位

datetime=2026-01-26 06:11:59,type=4,client=192.168.1.217,server=8.8.8.8,client_port=58356,server_port=53,class=IN,domain=www.google.com,flag_qr=0,dtype=A,dtype_num=1,inport=P1

datetime=2026-01-26 06:11:59,type=4,client=192.168.1.217,server=8.8.8.8,client_port=58356,server_port=53,class=IN,domain=www.google.com,flag_qr=1,dtype=A,dtype_num=1,answer=142.250.77.4,ttl=201,inport=P1

| Field | Description | 備註 | Format |
| --- | --- | --- | --- |
| type | Grism Syslog type | 4 | int |
| datetime | Date Time | Ex. 2018-08-08 17:15:10 | yyyy-mm-dd hh:mm:ss |
| client | Client IP address | Ex. 192.168.1.10 | IP address |
| server | Server IP address | Ex. 120.113.1.10 | IP address |
| client_port | Client port | Ex. 48573 | uint16 |
| server_port | Server port | Ex. 53 | uint16 |
| class | Dns class field | Ex. IN | string |
| domain | Dns domain field | Ex. google.com | string |
| flag_qr | Dns question/response flag | 0: question 1: response | 0/1 |
| dtype | Dns dtype field | Ex. A | string |
| opcode | Dns opcode field | Ex. QUERY, IQUERY, UPDATE | string |
| rcode | Dns rcode field | Ex. NOERROR, NXDOMAIN | string |
| update_op | Dns update operation field | Ex. ADD, DELETE, DELETE_ALL | string |
| record | Dns record field | Ex. www.example.com 300 IN A 1.2.3.4 | string |
| answer | Dns answer field | IP 或 domain | string |
| ttl | Dns ttl field | Ex. 600 | int |
| Inport | Input port | Ex. P1 | string |
| primary_server_name | SOA server name | Ex. ns1.gogrid.net | string |
| mailbox | SOA mail box | hostmaster.gogrid.net | String |
| sn | SOA sn | 2012042305 | string |

DPI log->HTTP

<134>datetime=2026-01-08.17:33:48,type=5,client=192.168.1.201,server=185.125.190.97,client_port=47148,server_port=80,method=GET,host=connectivity-check.ubuntu.com,uri=/,inport=P1

| Field | Description | 備註 | Format |
| --- | --- | --- | --- |
| type | Grism Syslog type | 5 | int |
| datetime | Date Time | Ex. 2018-08-08 17:15:10 | yyyy-mm-dd hh:mm:ss |
| client | Client IP address | Ex. 192.168.1.10 | IP address |
| server | Server IP address | Ex. 120.113.1.10 | IP address |
| client_port | Client port | Ex. 48573 | uint16 |
| server_port | Server port | Ex. 80 | uint16 |
| method | http method field | Ex. GET | string |
| host | http host field | Ex. google.com | string |
| uri | http uri field | Ex. /index.html | string |
| user_agent | http user agent field | Ex. chrome | string |

DPI log->SSL

<134>datetime=2026-01-08.17:25:54,type=6,client=172.104.105.194,server=192.168.1.207,client_port=32980,server_port=443,ja4s=t120200_c02b_8fe157b277d7,inport=P1

| Field | Description | 備註 |  | Format |
| --- | --- | --- | --- | --- |
| type | Grism Syslog type | 6 | int |  |
| datetime | Date Time | Ex. 2018-08-08 17:15:10 | yyyy-mm-dd hh:mm:ss |  |
| client | Client IP address | Ex. 192.168.1.10 | IP address |  |
| server | Server IP address | Ex. 120.113.1.10 | IP address |  |
| client_port | Client port | Ex. 48573 | uint16 |  |
| server_port | Server port | Ex. 443 | uint16 |  |
| Server_name | Ssl server_name field | Ex. google.com | string |  |
| ja3 | Ja3 string | Ex. 771,52244-52243-52245-49195-49199-158-49162-49172-57-49161-49171-51-156-53-47-10 | string |  |
| ja3_digest | Ja3 md5 hash | Ex. 39e62db039 deed96a9daf75dac dbd207 | String |  |
| Ja3s | Ja3s string | Ex. 771,4865,41-51-43 | String |  |
| Ja3s_digest | Ja3s md5 hash | Ex.2b0648ab686ee45e0e7c35fcfb0eea7e | String |  |
| Ja4 | Ja4 string | Ex.t13d171100_5b57614c22b0_3f5d972527c0 | String |  |
| Ja4s | Ja4s string | Ex. t120300_1301_ac87d89493dd | String |  |